Practical No:1

Study of Networking Devices

IP Address

Q1:- Identify the network address & subnet mask for the IP address 192.168.1.0

→ Given IP Address is 192.168.1.0
∴ The subnet mask is 255.255.255.0 (as it belongs to Class C)

In binary form:

11111111.11111111.11111111.00000000

Q2: Create 4 subnets

→ To create 4 subnets we need to borrow 2 bits from the host part
∴ 2² = 4

So, the new subnet mask is 255.255.255.192

Binary form:

11111111.11111111.11111111.11000000

:26 bits for the network part, 6 bits for the host part
 :Total number of subnets = 4

(Combining 2 or more subnets will give us the supernet.)

Total no of usable IP addresses in each subnet is 64 − 2 because 1 address is for network and 1 for broadcast so subtracting them.

62 IP addresses are usable.

CIDR format (Classless Inter-Domain Representation)
new subnet mask: 255.255.255.192/26

The 1st subnet is 192.168.1.0/26
The 2nd subnet is 192.168.1.64/26

subnet 1 = 192.168.1.0 → network address
usable host address: 192.168.1.62/26
broadcast address: 192.168.1.63/26

subnet 2 = 192.168.1.64
1st host address: 192.168.1.65
last host address: 192.168.1.126/26
broadcast address: 192.168.1.127

subnet 3 = 192.168.1.128
last address is 192.168.1.190
1st address is 192.168.1.129

subnet 4 = 192.168.1.192
last host address: 192.168.1.254
broadcast address: 192.168.1.255

Q.3 A network address 192.168.1.0/24
Create 3 subnet with subnet 1 consisting of 50, subnet 2 with 20 IP and subnet 3 with 10.

(Since the no of host bit is determine on IP address. IP address formula to calculate no of bits required to satisfy each subnet IP address is 2^n >= no of IP address required + 2
where n is the no of host bits and +2 represent n/ b)

Subnet 1 of 50 IP address
2^6 = 64 with 62 usable IP address
we need 6 host bits
Subnet mask is 255.255.255.192 (/26)

2^5 = 32 → 30 usable
Subnet mask is 255.255.255.224 (/27)

2^4 = 16
Subnet mask 255.255.255.240 (/28)

Subnet 1:
Subnet mask: 255.255.255.192 (/26)
Network address: 192.168.1.0/26
1st host address: 192.168.1.1/26
Last host address: 192.168.1.62/26
Usable IP address: 62
Broadcast: 192.168.1.63/26

Practical No:2

Demonstrate Static Routing

Router 0

Router 1

PC 0

PC 1

Demonstrate Routing Ospf in Packet tracer

Router 0

Router 1

Demonstrate Routing RIP in Packet tracer

Router 0

Router 1

PRACTICAL NO:3

Demonstrate Dhcp

Demonstrate Dns

Router 0

PC 0

PC 1

Demonstrate FTP

Router 0

Practical NO:4

Study of Cyptographhy-Substitution Techniques

Output:

PRACTICAL NO:5

Study of Symmetric Key

Demonstrate AES

Program:

Output:

Demonstrate DES

Output:

PRACTICAL NO:6

Study of Assymetric Key

Demonstrate RSA

Program:

Output:

PRACTICAL NO:7

Demonstrate MD5

Program:

Output:

PRACTICAL NO:8

Demonstrate Hash Function

Program:

Output:

PRACTICAL NO:9

To Create ,Export and validate a digital certificate

Output: